# Di Meng Xing MSPM0G3507 Quadruped Robot - Release Manual

Version: 2026-06-16  |  MCU: MSPM0G3507 (Cortex-M0+, 32MHz)  |  SDK: mspm0_sdk_1_30_00_03

# 1. Project Overview

Di Meng Xing is a quadruped robot based on TI MSPM0G3507, integrating voice recognition, OLED display, MP3 audio and servo gait control.

## Core Features

Voice command recognition (CI1302 offline module, no network required)

1.3-inch SH1106 OLED real-time status (128x64 monochrome)

WT2003H4 M09 MP3 module audio feedback

4-channel independent PWM servo (SG90-compatible), non-blocking gait engine

9 robot behaviors: Forward / Backward / Left Turn / Right Turn / Sit / Stand / Stop(Sleep) / Dance / Sing

# 2. Hardware Architecture

## 2.1 Core MCU - MSPM0G3507

| Item | Specification |
| --- | --- |
| Core | ARM Cortex-M0+ |
| Frequency | 32 MHz |
| Flash / SRAM | 128 KB / 32 KB |
| Dev Board | LP-MSPM0G3507 |
| SDK | mspm0_sdk_1_30_00_03 |
| Toolchain | Keil MDK V6.21 (ARMCLANG) |

## 2.2 Peripheral Modules

| Module | Model | Interface | Power |
| --- | --- | --- | --- |
| Voice Recognition | CI1302 | I2C bit-bang (PB2/PB3) | 3.3V |
| OLED Display | 1.3" SH1106 128x64 | SPI bit-bang (PA12/13/16/17/22) | 3.3V |
| MP3 Player | WT2003H4 M09 | UART1 (PA8/PA9, 115200) | External 5V |
| Servo x4 | SG90 Compatible | Independent PWM x4 | External 5V |
| Debug UART | USB-TTL | UART0 (PA10/PA11, 115200) | USB |

## 2.3 Complete Pin Map

| MSPM0 Pin | Connected To | Function | Notes |
| --- | --- | --- | --- |
| PA0 | Servo FR | PWM_2 (TIMA0 CCP0) | Front Right leg |
| PA8 | MP3 Module RX | UART1 TX | MCU -> MP3 |
| PA9 | MP3 Module TX | UART1 RX | MP3 -> MCU (optional) |
| PA10 | USB-TTL RX | UART0 TX | Debug printf output |
| PA11 | USB-TTL TX | UART0 RX | Debug input |
| PA12 | OLED D0 | SPI SCL (bit-bang) | Clock |
| PA13 | OLED D1 | SPI SDA (bit-bang) | Data |
| PA14 | On-board LED | GPIO Output | Status indicator (active low) |
| PA15 | Servo FL | PWM_1 (TIMA1 CCP0) | Front Left leg |
| PA16 | OLED DC | GPIO Output | Command/Data select |
| PA17 | OLED RES | GPIO Output | Reset |
| PA21 | Servo RL | PWM_3 (TIMG6 CCP0) | Rear Left leg |
| PA22 | OLED CS | GPIO Output | Chip Select |
| PA23 | Servo RR | PWM_4 (TIMG0 CCP0) | Rear Right leg |
| PB2 | CI1302 SCL | I2C SCL (bit-bang) | Voice module clock |
| PB3 | CI1302 SDA | I2C SDA (bit-bang) | Voice module data |

## 2.4 Power Distribution

External 5V (independent, >= 2A)
    +-- Servo VCC x4  <-- MUST use independent power!
    +-- MP3 Module VCC
    +-- DO NOT connect to board 5V pin! (will backfeed and damage PSU)

Board 3.3V (USB or on-board LDO)
    +-- OLED VCC
    +-- CI1302 VCC

ALL modules GND must share common ground.

!! Servos MUST use independent power. 4 servos peak ~2-3A. Drawing from board 5V causes voltage sag, screen brownout, and MCU reset.

# 3. Software Architecture

## 3.1 Source Tree

driverlib/empty/
+-- empty.c                    # Main program (single-file, ~720 lines)
+-- board.c / board.h          # Board init + delay_us/ms
+-- ti_msp_dl_config.c/.h      # SysConfig auto-generated
+-- empty.syscfg               # SysConfig project config
+-- LCD/
    +-- lcd_init.c/.h          # OLED driver (SH1106 SPI + framebuffer)
    +-- lcd.c/.h               # Draw primitives (char/line/image)
    +-- lcdfont.h              # ASCII fonts (12/16pt)
    +-- img_idle.h             # Idle image
    +-- img_go.h / img_back.h  # Forward/Backward
    +-- img_left.h / img_right.h # Left/Right turn
    +-- img_sit.h / img_walk.h # Sit/Walk
    +-- img_sleep.h / img_stop.h # Sleep/Stop
    +-- img_dance.h / img_sing.h # Dance/Sing
+-- keil/                      # Keil project files

## 3.2 Startup Flow

Power On
  +-- board_init()             # Clocks / SysTick / GPIO / UART
  +-- MP3_SetVol(25)           # MP3 volume 25/31
  +-- MP3_Stop()               # Ensure MP3 stopped
  +-- OLED_Init()              # SH1106 init + clear screen
  +-- Servo_Init()             # 4 servos to mid (CCR=750)
  +-- Self-test                # Servo wiggle to confirm PWM OK
  +-- Main Loop ---------------+
        +-- CI1302_Recognition() # Poll voice result
        +-- New cmd -> HandleCommand()
        |   +-- Update OLED
        |   +-- Trigger MP3
        |   +-- CI1302 speak
        |   +-- Set state (g_state)
        +-- Non-blocking gait update (per g_state)
        +-- delay_ms(5) -------+

## 3.3 State Machine

| State | Enum | Trigger (cmd ID) | Gait Behavior |
| --- | --- | --- | --- |
| STATE_IDLE | 0 | 0x07 Stop | No gait update |
| STATE_WALK_FORWARD | 1 | 0x01 / 0x06 | Non-blocking forward trot |
| STATE_WALK_BACKWARD | 2 | 0x02 | Non-blocking backward trot |
| STATE_TURN_LEFT | 3 | 0x03 | Non-blocking left pivot |
| STATE_TURN_RIGHT | 4 | 0x04 | Non-blocking right pivot |
| STATE_SIT | 5 | 0x05 | Blocking: sit pose |
| STATE_DANCE | 6 | 0x08 | Non-blocking 4-phase wave |

# 4. Communication Protocols

## 4.1 CI1302 Voice Recognition (I2C)

| Item | Value |
| --- | --- |
| Interface | Bit-bang I2C (PB2=SCL, PB3=SDA) |
| Slave Address | 0x34 (7-bit) |
| Result Register | 0x64 |
| Speak Cmd Register | 0x6E |

Voice Command ID Map:

| Voice Command | Cmd ID | Function |
| --- | --- | --- |
| "Forward" | 0x01 | Walk forward |
| "Backward" | 0x02 | Walk backward |
| "Left turn" | 0x03 | Pivot left |
| "Right turn" | 0x04 | Pivot right |
| "Sit" | 0x05 | Sit pose |
| "Walk" | 0x06 | Walk forward (same as 0x01) |
| "Stop" | 0x07 | Sleep pose |
| "Dance" | 0x08 | Dance gait |
| "Sing" | 0x09 | MP3 sing only |

## 4.2 WT2003H4 MP3 (UART)

| Item | Value |
| --- | --- |
| Interface | UART1 (PA8=TX, PA9=RX) |
| Baud Rate | 115200 bps |
| Protocol | Binary: 7E + len + cmd + params + checksum + EF |

Key Macros:

MP3_Play(n)   // Play track N (0xA0 cmd + 2-byte index)
MP3_Stop()    // Stop playback (0xAB cmd)
MP3_SetVol(v) // Volume 0-31 (0xAE cmd)

// Baud config: UART1 4MHz -> 115200
DL_UART_Main_setBaudRateDivisor(UART_1_INST, 2, 11);

## 4.3 SH1106 OLED (SPI)

| Item | Value |
| --- | --- |
| Resolution | 128x64 monochrome |
| SPI Pins | PA12=SCL, PA13=SDA (bit-bang) |
| Control Pins | PA17=RES, PA16=DC, PA22=CS |
| Framebuffer | OLED_GRAM[128][8] (1024 bytes) |
| Addressing | Page mode (SH1106 no horizontal) |
| Image Format | Image2Lcd: horiz scan, mono, 6-byte header |

# 5. Servo System

## 5.1 PWM Parameters

| Parameter | Value | Description |
| --- | --- | --- |
| PWM Frequency | 50 Hz | Standard servo frequency |
| Timer Clock | 500 kHz | After prescaler |
| PWM Period (ARR) | 10000 | 20ms period |
| CCR Range | 250 ~ 1250 | 0.5ms ~ 2.5ms pulse width |
| Neutral CCR | 750 | 1.5ms pulse = 90 degrees |
| CCR <-> Angle | CCR = 250 + angle * 1000 / 180 | Linear mapping |

## 5.2 Servo Mapping & Mount Direction

| Ch | Leg | PWM Instance | MCU Pin | Mount | g_legInv |
| --- | --- | --- | --- | --- | --- |
| ch0 | FL (Front Left) | PWM_1 (TIMA1) | PA15 | Gear forward (normal) | 0 |
| ch1 | FR (Front Right) | PWM_2 (TIMA0) | PA0 | Gear forward (normal) | 1 |
| ch2 | RL (Rear Left) | PWM_3 (TIMG6) | PA21 | Gear backward (flipped) | 0 |
| ch3 | RR (Rear Right) | PWM_4 (TIMG0) | PA23 | Gear backward (flipped) | 1 |

Inversion Logic (g_legInv):

inv=0 (normal):    CCR = v           // direct gait curve value
inv=1 (inverted):  CCR = 1500 - v    // mirrored around neutral 750

FR and RR have their servo gears facing opposite to the other legs. Same CCR value would produce opposite physical motion. Inversion unifies the physical direction across all 4 legs.

## 5.3 Special Pose Reference

| Pose | FL (ch0) | FR (ch1) | RL (ch2) | RR (ch3) |
| --- | --- | --- | --- | --- |
| Neutral (stand) | 750 | 750 | 750 | 750 |
| Sit | 750 | 750 | 1250 | 250 |
| Sleep (lie down) | 250 | 1250 | 1250 | 250 |

# 6. Gait Engine

## 6.1 Core Parameters

#define CYCLE_STEPS     100    // Steps per cycle (100 x 5ms = 500ms/cycle)
#define CYCLE_DELAY_MS  5      // Main loop delay (ms)
#define SMOOTH_STEP     30     // CCR approach per step (higher = faster)
#define LAG_STEPS       (CYCLE_STEPS / 2)  // Diagonal leg phase offset = 50 steps = half-cycle

## 6.2 Gait Curve Formulas

Forward Curve Gait_ForwardCCR(t):

t %= 100;
if (t < 10)
    return 1050 - t * 60;    // 10%: fast forward swing (1050->450)
else
    return 450 + (t-10) * 6; // 90%: slow backward push (450->990)

Backward Curve Gait_BackwardCCR(t):

t %= 100;
if (t < 10)
    return 450 + t * 60;     // 10%: fast backward swing (450->1050)
else
    return 1050 - (t-10) * 6;// 90%: slow forward push (1050->510)

Design: Asymmetric duty cycle (10% fast swing + 90% slow push) mimics biological gait. Fast swing minimizes air time; slow push provides sustained thrust.

## 6.3 Five Gait Modes

| Gait Mode | Diagonal Pairing | Description |
| --- | --- | --- |
| Forward GAIT_FORWARD | FL+RR lead / FR+RL lag (half-cycle) | Diagonal trot, diagonal legs move together |
| Backward GAIT_BACKWARD | FL+RR lead / FR+RL lag (half-cycle) | Reverse diagonal trot |
| Left Turn GAIT_TURN_LEFT | FL+RL backward / FR+RR forward | Right pushes forward + left backward = CCW rotation |
| Right Turn GAIT_TURN_RIGHT | FL+RL forward / FR+RR backward | Left pushes forward + right backward = CW rotation |
| Dance GAIT_DANCE | FL->FR->RL->RR each 1/4 cycle apart | 4-phase wave + front/rear phase inversion |

## 6.4 Smooth Interpolation

All non-blocking gaits use the same interpolation to avoid servo jitter from instant CCR jumps:

for (int ch = 0; ch < 4; ch++) {
    uint16_t cur = g_servoCCR[ch];
    if (cur < target[ch])      { cur += SMOOTH_STEP; if (cur > target[ch]) cur = target[ch]; }
    else if (cur > target[ch]) { cur -= SMOOTH_STEP; if (cur < target[ch]) cur = target[ch]; }
    g_servoCCR[ch] = cur;
    Servo_WriteCCR(ch, cur);
}

# 7. Build & Flash

## 7.1 Project Configuration

| Item | Value |
| --- | --- |
| Project File | keil/empty_LP_MSPM0G3507_nortos_keil.uvprojx |
| Compiler | ARMCLANG V6.21 |
| Optimization | -O0 (debug) |
| Code Size | ~14 KB Flash |
| Pre-build | SysConfig auto-generates ti_msp_dl_config.c/.h |

## 7.2 Include Paths

..                    # empty directory
..\..\..             # SDK root
..\LCD                # OLED driver + images

## 7.3 First Power-On Checklist

USB-TTL connected to PA10/PA11, 115200bps. Should see boot banner and [SYS] Ready.

OLED shows idle image (gImage_idle).

Servos briefly wiggle then return to neutral (self-test).

Speak "Forward" / "Backward" etc. to test each function.

# 8. Voice Command Quick Reference

| Command | ID | OLED | Servo Action | MP3 |
| --- | --- | --- | --- | --- |
| "Forward" | 0x01 | Forward icon | Continuous forward trot | Track 1 |
| "Backward" | 0x02 | Backward icon | Continuous backward trot | Track 2 |
| "Left turn" | 0x03 | Left icon | Continuous left pivot | Track 3 |
| "Right turn" | 0x04 | Right icon | Continuous right pivot | Track 4 |
| "Sit" | 0x05 | Sit icon | Front legs neutral + rear fold 90 deg | - |
| "Walk" | 0x06 | Walk icon | Return to neutral (stand) | - |
| "Stop" | 0x07 | Sleep icon | Neutral -> delay -> sleep pose | Track 2 |
| "Dance" | 0x08 | Dance icon | Continuous 4-phase wave | - |
| "Sing" | 0x09 | Sing icon | No movement (audio only) | Track 1 |

# 9. Known Issues & Resolution Log

## Resolved (11 items)

| Issue | Root Cause | Solution |
| --- | --- | --- |
| OLED no display | Code was for ST7789 TFT | Rewrote SH1106 init + page addressing |
| OLED garbled | SH1106 lacks horizontal addressing | Use page addressing in OLED_Refresh() |
| Servo not moving | Wrong CCR range 0-3750 | Corrected to 250-1250 (0.5-2.5ms) |
| Code >32KB | Keil eval limit | Disabled Chinese fonts + large ASCII |
| Rear legs wrong direction | Gear mounted backward | Added g_legInv[4] inversion flags |
| Dance: rear legs stuck | 90-CCR() = negative overflow | 4-phase wave + g_legInv fix |
| Screen flicker | Missing debounce | Added g_lastDisplayedCmd cache |
| EPERM build error | syscfg.bat permission | Disabled pre-build step |
| Undeclared identifier | Missing img_sleep/dance/sing.h | Created placeholder image headers |
| Stop action unnatural | Direct servo center | Center -> delay -> sleep pose |
| Include path conflict | Duplicate board.h | Cleaned redundant paths |

## Pending (5 items)

| Issue | Status | Suggestion |
| --- | --- | --- |
| Forward walking not actually moving | Needs tuning | Adjust gait curve, LAG_STEPS, CYCLE_STEPS |
| Dance gait | New, untested | Needs hardware verification |
| Left/Right turn | New, untested | Needs hardware verification |
| OLED images are blank | Placeholders | Use Image2Lcd to generate real images |
| SMOOTH_STEP=30 too large | May skip smoothing | Try 3-10 for smoother motion |

# 10. Appendix

## A. MP3 TF Card File Naming

Place files in TF card root. Name as 001xxx.mp3 ~ 008xxx.mp3, corresponding to MP3_Play(n):

| MP3_Play(n) | Suggested Track |
| --- | --- |
| 1 | Forward / Sing sound |
| 2 | Backward / Stop sound |
| 3 | Left turn sound |
| 4 | Right turn sound |
| 5 | Sit sound |
| 6 | Walk sound |
| 7 | Dance music |
| 8 | Sing music |

## B. OLED Image Creation (Image2Lcd)

Prepare 128x64 or smaller B/W image (BMP/PNG)

Open Image2Lcd -> Import image

Settings: Horizontal scan, Monochrome, No compression

Generate C array -> Replace array content in LCD/img_*.h

Array name must match the include variable name (e.g. gImage_sleep)

## C. SH1106 vs SSD1306 Key Differences

| Item | SSD1306 | SH1106 (This Project) |
| --- | --- | --- |
| Column start | 0 | 2 (need +2 offset) |
| Horizontal addressing | Supported | NOT supported |
| Page addressing | Supported | Supported (used here) |
| Charge pump register | 0x8D | 0xAD |
| Max columns | 128 | 132 |

## D. Debug Serial

Connect PA10/PA11 (115200bps). Observable output:

[CMD] 0xXX <Name> - Each voice recognition result

[DBG] g_state=X - Current state machine state

Servo CCR values (uncomment printf to observe)

## E. Project Version History

| Version | Display | Notes |
| --- | --- | --- |
| V1 (empty.txt) | TJC serial screen Plan A (page switch) | Verified working |
| V2 (empty.c old) | TJC serial screen Plan B (widget update) | Flicker fixed |
| V3 (main.c) | TJC serial screen, modularized | Wrong cmd IDs (0x01 vs 0x81) |
| V4 (current) | SH1106 OLED monochrome | Current release version |

------------------------------------------------------------

- End of Document -

Update this document when hardware wiring, voice commands, or gait parameters change.